Project: Customizable Telematics Platform

The general purpose of the finished product is to be a customizable telematics and datalogging platform based on the MBED LPC1768 microcontroller, assembled in a rugged enclosure. The device will have several standard hard-wired I/O, as well as some configurable I/O and spare circuits. The usage of this device is primarily to assist with troubleshooting intermittent issues on fire apparatus in near real-time, as well as provide feedback on vehicle usage.

Scope of work

The scope of work includes all electrical circuit and PCB design, including the proper selection of components to meet the specifications below. 10 completed boards will be provided by the contractor, unpopulated with components. The contractor is not required to provide any components. Documentation including schematics, PCB board layout files, and bill of materials are to be provided.

Contractor expectation

- Design is “best-effort”, since the scope is to provide the ability to build, not a built and proven unit.

  - XBEE module interface requirements will be studied from the data sheets

- Verification, which may include test laboratory involvement, is not in scope. Examples:

  - EMI/EMC

  - Load Dump

  - Reverse Voltage Protection

  - Temp/Humidity/Vibration

- Environmental

  - The Deutsch module is a sealed module, until you poke a hole for the antenna

  - Temperature range – typical industrial/automotive: -40 to +85C. Some environments may require a higher range typically due to heat-loading (under hood, or elsewhere that sun load is very high)

  - Temp range for the LPC 1768 module – I haven’t looked online to see if they spec’d industrial parts or if some are commercial only (0 to 70C).

  - Vibration: better sockets for the mbed and the XBEE parts, nylon tie-strap to fix them in their socket.

- Consider providing the contractor 1 to 2 units for HW checkout / modifications.

Basic functional features

The completed device will contain the following functions/features:

(2) CAN channels, accessible through the device connector

No internal terminating resistors

(1) J1708/J1587 Channel, accessible through the device connector

(1) RS232 channel, accessible through the device connector

(1) Micro SD card, not accessible externally

(1) socket for MBED LPC1768 microcontroller

(1) socket for XBEE Wi-Fi or XBEE Cellular Module (datasheets attached, samples available)

Reverse polarity protection

Load dump protection

Overvoltage protection

(1) LED indicator for Power

(2) LED indicators controlled by microcontroller

Ability to monitor module battery voltage using analog input and voltage divider

(4) General purpose I/O, configurable using jumpers and software, with the following capabilities:

Grounding Input

12 volt input

500 mA sinking output

(3) 5 volt analog inputs

5 volt power and ground accessible externally for analog sensors

Candidate pin assignments from function list above (note pins 9,10 are the additional CAN channel).

| Pin | Function |  | Function | Pin |
| --- | --- | --- | --- | --- |
| 1 | Gnd |  |  | 40 |
| 2 | Vin |  |  | 39 |
| 3 | VBat(RTC) |  |  | 38 |
| 4 | /Reset |  |  | 37 |
| 5 | MicroSD |  |  | 36 |
| 6 | MicroSD |  |  | 35 |
| 7 | MicroSD |  |  | 34 |
| 8 | MicroSD(Select) |  |  | 33 |
| 9 | CAN 1 |  |  | 32 |
| 10 | CAN 1 |  |  | 31 |
| 11 |  |  | CAN 2 | 30 |
| 12 |  |  | CAN 2 | 29 |
| 13 | RS232 |  | J1708 | 28 |
| 14 | RS232 |  | J1708 | 27 |
| 15 |  |  | GPIO4 | 26 |
| 16 | Ain 5 |  | GPIO3 | 25 |
| 17 | Ain 4 |  | GPIO2 | 24 |
| 18 | Ain 3 |  | GPIO1 | 23 |
| 19 | Ain 2 |  | LED2 | 22 |
| 20 | Ain 1 (Batt) |  | LED1 | 21 |

- XBEE Modules (from notes further below, these require 4 pins)
2 logic signals, and a UART pair Rx/Tx.

General Construction

For reference, the terms Front, Back, Top, Bottom, Left, and Right will be used, and refer to the following:

Bottom – the edge of the board closest to the enclosure connectors

Top – the edge of the board opposite of the enclosure connectors

Front – the surface of the PCB that the majority of the components mount to

Back – the “solder side” of the board

Left / Right – as viewed from the front of the board, connectors toward the bottom.

Any unused pins for the modem socket, MBED, or enclosure connectors should be connected to plated though holes in the PCB for future use.

Any unused space on the PCB should be populated with pads plated through holes on a 100 mil grid for future use.

All components should be through-hole when possible SMT components are allowed as needed, as long as it is possible to install by hand.

Three or Four holes that will fit a #4 machine screw shall be provided on the PCB for applications where the PCB is mounted to a surface rather than within the Deutsch enclosure.

The PCB should utilize double sided construction, if possible, to allow modification of traces for application specific situations.

The PCB should be supplied with standard green solder mask and silkscreen.

Enclosure

The enclosure will be made of the following two Deutsch components:

| Description | Deutsch P/N | Digikey P/N |
| --- | --- | --- |
| Enclosure | EEC-325X4A-E016 | 1734-1063-ND |
| Connector Base | DTM13-12PA-12PB-R008 | 1734-1131-ND |

A sample of both components can be supplied for reference.

External Connections

Below is a table showing the preferred pinout for external connections. It would be preferred to maintain at least the gray connector pinout in order to match existing modules.

| Grey Connector |  |  | Black Connector |  |
| --- | --- | --- | --- | --- |
| 1 | Spare |  | 1 | Analog Ground |
| 2 | Spare |  | 2 | 5 volt Analog Supply |
| 3 | Spare |  | 3 | Digital Input / Output 1 |
| 4 | RS232GND |  | 4 | Digital Input / Output 2 |
| 5 | RS232TX |  | 5 | Digital Input / Output 3 |
| 6 | RS232RX |  | 6 | Digital Input / Output 4 |
| 7 | CAN2 Low |  | 7 | J1708A |
| 8 | CAN2 High |  | 8 | J1708B |
| 9 | CAN1 Low |  | 9 | Analog in 1 |
| 10 | CAN1 High |  | 10 | Analog In 2 |
| 11 | Module Ground |  | 11 | Analog in 3 |
| 12 | Module Battery |  | 12 | Analog In 4 |

The antenna port for the Wifi or Cellular module will exit the enclosure through a drilled hole, opposite of the end that the connectors are on. The antenna will have a 6” lead for attaching to the u.fl connector on the modem.

Wi-Fi/Cellular Socket

There should be one socket, for mounting either an XBEE/Digi Wi-Fi Modem or Cellular Modem. The Wi-Fi and cellular modems are 20 pin devices with compatible pinouts. The socket should be mounted toward the top of the PCB. The pins used in this application are as follows:

| 1 | VCC/Power |  | 11 |  |
| --- | --- | --- | --- | --- |
| 2 | Data Out (connect to mbed serial in) |  | 12 |  |
| 3 | Data In (connect to mbed serial out) |  | 13 |  |
| 4 | DIO12 / TCP Established (mbed digital I/O) |  | 14 |  |
| 5 |  |  | 15 | DIO5 / Associate (mbed digital I/O) |
| 6 |  |  | 16 |  |
| 7 |  |  | 17 |  |
| 8 |  |  | 18 |  |
| 9 |  |  | 19 |  |
| 10 | Ground |  | 20 |  |

Unused pins will still be accessible with a plated hole adjacent to the socket. A power supply shall be included in the design with sufficient power to run the modem, without causing undesired voltage drops at other parts of the device.

The part numbers used for the modems are as follows:

| Modem | Digi Part number | Digikey Part Number |
| --- | --- | --- |
| Wi-Fi | XB2B-WFUT-001 | 602-1378-ND |
| Cellular | XB3-C-A1-UT-001 | 602-2211-ND |

Both modems can be supplied for reference.

MBED Socket

The socket will be a double row of pins, for ease of testing and possible installation of a daughterboard if the PCB is not mounted within the enclosure. The socket for the MBED LPC1768 should be located in a way that the USB connector is accessible when the PCB is removed from the enclosure without having to remove the antenna connector.