# Power Lock Data Sheet and Programmers Reference

Preliminary Data Sheet

This preliminary data sheet is for review only. 
The manufacturer reserves the right to change characteristics without notice.

# General

The 5-wire Power Lock interface supports LIN BUS.

## Reference Documents

The following documents and referenced sections are applicable to this design.

| Document | Section | Description |
| --- | --- | --- |
| SCH_11377 | v1.0 | Schematic |
| JDQ 53.3 |  | Level 3 Device Requirements |
| JDQ 53.3 | 9.2.3.10 | Test Pulse for Clamped load dump |

# Electrical Ratings

Ratings are based on characterization of components and should be verified in end-use environment.

All voltages with respect to Ground (unless otherwise noted);

| Pos | Parameter | Symbol | Limit Values |  | Unit | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |  |
| Voltages |  |  |  |  |  |  |
| 1 | Un-switched Battery | Battery (VBat) BatteryMAX BatteryJUMP BatteryOPER BatteryOFF | -56 - 10 0 | 60 36 32 6 | V V V V | Load Dump Jump Start 5 min Operating range Device in reset/off |
| 2 | Wake-In Active Wake-In Inactive Wake-In | Wake-In Wake-InMAX Wake-InH Wake-InL | -40 +4.0 -40 | 40 40 1.6 | V V V | Max without damage Actively wakes device Does not wake device |
| 3 | Wake-Out Active Wake-Out Inactive Wake-Out | Wake-Out Wake-OutH Wake-OutL | >VBat – 2 - | - | V V | Wake-Out does not have active pull-down. |
| 6 | LIN BUS | LINMAX LINREC LINACT | -40 Vs x 0.8 - | 40 Vs Vs x 0.2 | V V V | External fault applied Recessive Active |
| Currents |  |  |  |  |  |  |
| 1 | Module Current Nominal Low Power Un/Lock Active | IBATTERY IBATTERY-NOM IBATTERY-LOW IBATTERY-ACT | 2 - 1.0 | 20 0.7 2.5 | mA mA A | Operating Sleep Mode During Un/Lock pulse |
| 2 | Current on Wake-In | IWAKE-IN | 4 8 |  | mA mA | Contact wetting current 12v applied to pin 24v applied to pin |
| 3 | Wake-Out Output Current | IWAKE-OUT IWAKE-OUT-ACT IWAKE-OUT-OFF | 0.2 - | 100 | A µA | self-limited leakage current |
| Temperature |  |  |  |  |  |  |
|  | Operating | TOPER | -40 | +85 | °C |  |
|  | Storage | TSTORAGE | -55 | +105 | °C | JDQ53.3 Level 2 |
| Temperate Exceptions |  |  |  |  |  |  |
|  | Mallory PK-12N40PQ | Acoustic Signal | -20 | +90 | °C | Operating Temp Range |
|  |  |  | -30 | +100 | °C | Storage Temp Range |
|  | Mabuchi FC280SC-20150 | Motor | -30 | +80 | °C | Component Rating |
| Timing |  |  |  |  |  |  |
|  | Tolerance | Timing-NOM | -1 | +1 | % | At 25 °C |
|  |  | Timing-TEMP | -5 | +5 | % | -40°C to +85°C |